School of Surveying & Built Environment

Total Station Differential Levelling Procedure – Topcon Field

V1.1

# Document History

| Date | Version | Issue | Amendments | Author(s) |
| --- | --- | --- | --- | --- |
| 18/07/2024 | 0 | 1 | Document Created | CMcA & KZ |
| 14/03/2025 | 1 | 1 | Update for Magnet to Topcon name change | CMcA |

- Contents

Document History	2

1.	Introduction	4

2.	Point naming conventions	5

3.	Example run	6

4.	Configuring Topcon Field	8

4.1.	Establishing a new job	8

4.2.	Instrument settings	8

4.3.	Setup menu	13

5.	Starting the Differential Levelling Survey	16

5.1.	Initial Occupation	16

5.2.	Observing the Foresight	19

5.3.	Moving to a new Occupation	20

5.4.	Downloading your job file	21

6.	Uploading TS Differential Levelling to Topcon Tools	22

6.1.	Initial Import into Topcon Tools	22

6.2.	Averaging observations in a Spreadsheet	24

6.3.	Example calculations	24

6.4.	Entering your level data into Topcon Tools	25

7.	Appendix A: Manual Entry of Levelling Data into Magnet/Topcon Tools	26

# Introduction

This document has been prepared to provide a practical explanation and instructions for differential levelling using TOPCON FIELD with a TOPCON Total Station and a fixed height pole.

The overview of this technique is outlined in the “Total Station Differential Levelling Procedure” document available on GitHub, available here: https://github.com/UniSQ-Surveying/Differential_Levelling

It is based on the process outlined in the Special Publication 1 v2.2 Guideline for Differential Levelling Section 3.2. The Guideline is available on the ICSM website here.

This method out lined in this document was developed by UniSQ student Kristy Zemski as part of her University studies, under the guidance of Queensland Department of Resources Geodetic Surveyor, Garry Cislowski, and UniSQ Professional Fellow (Surveying), Chris McAlister. Contributions were also made by Joe Culliver, Jordan Williams, Damian Forknall and Andrew Cleland.

This document, along with additional resources, can be downloaded from the UniSQ Surveying GitHub.

This document will be updated over time, however if you have feedback or comments, please contact Chris McAlister at chris.mcalister@unisq.edu.au

# Point naming conventions

Before commencing the configuration of the Total Station and job settings, it is critical to understand the point naming conventions used throughout this process.

The Total Station will never be set up over a mark (as is the case with traditional differential levelling) so the naming of the Total Station occupation is simply:

- OCCX

  - Where “X” is the number occupation the Total Station is at. E.g. “OCC1” is used for Total Station setup 1, “OCC2” for 2 and so on.

This occupation number is then used as the prefix for the backsight and foresight observation numbering, which are three digits, such as:

- Backsight: 101

  - Where the first “1” indicates the occupation number

  - The next two digits “01” indicate the mark that the Total Station is observing to – in this example our first backsight from occupation 1

- Foresight: 102

  - Again, “1” indicates the occupation number

  - “02” indicates the second mark observed – our first foresight

At the second occupation our numbers will be:

- Occupation: OCC2

- Backsight: 202

  - This indicates we are observing to mark “02”, our previous foresight

- Foresight: 203

  - This indicates we are observing to mark “03”, our new foresight

This pattern continues until the survey is complete, noting that the point numbers will also be used on the reverse run.

Drawing a field note sketch of your level run as you go will be helpful to maintain point numbering conventions.

# Example run

Forward Run: First set up

| PSM123 | BS | OCC1 (TS) | FS | CP1 |
| --- | --- | --- | --- | --- |
| 101 |  |  |  | 102 |

  - TS Occupation is called OCC1

  - Backsight is set to point name: PSM123

  - Level observations to backsight point are called 101

  - Level observations to foresight point are called 102

  - Foresight is set to point name: CP1

  - Move TS to next occupation OCC2

Forward Run: Second set up

| CP1 | BS | OCC2 (TS) | FS | CP2 |
| --- | --- | --- | --- | --- |
| 202 |  |  |  | 203 |

  - TS Occupation is called OCC2

  - Backsight is set to point name: CP1

  - Level observations to backsight point are called 202

  - Level observations to foresight point are called 203

  - Foresight is set to point name: CP2

  - Move TS to next occupation OCC3

Return Run: First set up

| CP2 | BS | OCC3 (TS) | FS | CP1 |
| --- | --- | --- | --- | --- |
| 303 |  |  |  | 302 |

  - TS Occupation is called OCC3

  - Backsight is set to point name: CP2

  - Level observations to backsight point are called 303

  - Level observations to foresight point are called 302

  - Foresight is set to point name: CP1

  - Move TS to next occupation OCC4

Return Run: Second set up

| CP1 | BS | OCC4 (TS) | FS | PSM123 |
| --- | --- | --- | --- | --- |
| 402 |  |  |  | 401 |

  - TS Occupation is called OCC4

  - Backsight is set to point name: CP1

  - Level observations to backsight point are called 402

  - Level observations to foresight point are called 401

  - Foresight is set to point name: PSM123

# Configuring Topcon Field

## Establishing a new job

- Open Topcon Field

- Connect your controller to your Total Station via Bluetooth

- Create a new job for your Differential Levelling with a Total Station job. The software will treat your level run as a traverse. If you accidentally combine your level run with actual traverse data it is very difficult to untangle and will confuse the software immensely!

## Instrument settings

- From the main menu, selection Configure, the Survey

Select Edit in the Optical Configuration section, as shown below.

Change the Name field to TS DIFF LEVEL with the Type as Robotic. Click Next.

On the Instrument screen, set the Manufacturer and Model as appropriate. Then enter the following values for Height of Reflector (HR) and Height of Instrument (HI), then click Next when complete.

  - Foresight HR = 0.000m

  - Backsight HR = 0.000m

  - HI: 0.000m

On the Connection Mode screen select Bluetooth TS and click Next.

On the Search/Track screen accept the default settings and click Next

On the Survey Settings screen 1, set the following values, then click Next.

- Meas method: Direct/Reverse (this is Topcon speak for FL/FR)

- Search After Turn: Search and Aim

- Set tolerance as appropriate for your job

On the Survey Settings screen 2, set the following values, then click Next.

- Measurement type: HA/HD/VD (this assists with display options only)

- Set BS and FS Target to ATP1 360 (or other prism being used)

On the Survey Settings screen 3, set the following values, then click Next.

- Select Precise EDM mode, and

- Set EDM Mode to Fine

On the Auto Topo screen click Next.

On the Monitor Options screen, set the following values, then click Next.

- Measurement method: Direct/Reverse

- Sets: 1

- Ensure “Store as checkpoint” is not ticked

On the Topo Output Config screen, click Next.

On the Stake Settings screen 1, click Next.

On the Stake Settings screen 2, click Next.

On the Grade Stake Marking, click Next.

On the Stake Settings screen 3, click Next.

On the Staked Point Icon screen, click Next.

On the Point Naming screen, set the following values, then click Next.

  - Increment Point Number: 0

  - Prefix Suffix: None

On the Miscellaneous screen, select the following settings as ticked as a minimum:

  - Right/Left Rod to TS

  - Beep on Storing Points

  - Automatically display BS Setup

  - Remember Occ/BS if set

  - Prompt for Travers Advance

Click the Green Tick in the top right corner.

Click the Green Tick in the top right corner of the Job Configuration screen.

Note that your Total Station might disconnect and then prompt you to reconnect to the controller at this point. This is normal, just connect as previously.

## Setup menu

- Select the Setup icon on the main screen (tripod icon), then Backsight.

- On the Backsight screen:

  - Enter the Occupation Point information as below

  - Enter the Backsight Point as the actual name/number for the mark you are using in your survey.

  - Click Next

- You will receive a Warning screen telling you the Occupation point has not been found. Click Next.

- On the Add Point screen, enter arbitrary values for East, North and Elevation. You can take a photo of your set up using the Photo tab if you like. Then click the Green Tick button.

You may receive a Warning screen telling you the Backsight point has not been found. If so, click Next.

  - On the Add Point screen, enter arbitrary values for East, North and Elevation that are different to your Occupation Point coordinates!

  - Add photo of your set up and mark using the Photo tab

  - Then click the Green Tick button.

This will take you to a Backsight Measurement screen.

  - Check that the heading at the top of the screen shows Backsight -Direct/reverse: Normal.

    - If it doesn’t, select the Cogs symbol button near the top right and change the settings as per the instructions in Section 4.2, Point 7 onwards.

  - Enter an approximate Magnetic Bearing into Set Circle to from your Total Station to the Backsight.

  - Centre the Total Station crosshairs on the prism at the Backsight (remember to do your parallax error checks!)

  - Push the Set Button

  - It will then prompt you to measure your Backsight in Reverse (FR)

  - Centre the Total Station crosshairs on the prism at the Backsight (remember to do your parallax error checks if you have changed user.)

  - Select the Measure button in the bottom right.

The Backsight-Direct/Reverse screen 1 will open, giving you the information from your Backsight observations. If they are acceptable, click Accept.

The Backsight-Direct/Reverse screen 2 will open, giving you the information from your Backsight observations. Click the Home button (the blue house icon) in the top right corner.

You will be returned to the main menu.

# Starting the Differential Levelling Survey

## Initial Occupation

- Commence the survey by selecting Survey then Topo

- This will open the measurement screen. It should say Sideshot-Direct/Reverse Normal at the top of the screen.

  - Topcon uses the term Sideshot for any points that are not Control points (e.g. BS and FS points).

  - In this measure screen you have the option to measure Sideshots or Traverse points.

  - To change between them, select the button in the top left (this will be an M in Magnet branded versions, or a T in Topcon branded versions), then select Measure from the drop-down menu, and select the measurement option you wish to use.

  - You should use Sideshot mode for measuring your Levelling observations, and Traverse mode for your Backsight and Foresight measurements.

- In Sideshot mode:

  - Complete the Point numbering according to the convention outlined in Section 2. Enter the point number as a three digit number with the first number being the occupation point number and the final two numbers being the point increment number e.g.: 101 – 1 being the occupation point and 01 being the observation point number.

  - Ensure your height is set to 0.000m

  - Centre the Total Station crosshairs on the prism at the Backsight in FL.

  - Press the Measure button in the bottom right (the Total Station), or the measure and save button (Total Station with the green tick).

- You will then be prompted to measure the Backsight in Reverse (FR).

  - If you are using ATR, the instrument will automatically turn to FR and aim at the prism. Check the crosshairs are centred on the prism using the telescope prior to undertaking any measurements.

  - If you are not using ATR, turn into FR and centre on the prism.

  - Press the Measure button in the bottom right (the Total Station), or the measure and save button (Total Station with the green tick).

The Sideshot-Direct/Reverse screen 1 will open, giving you the information from your Backsight observations. If they are acceptable, click Accept.

The Sideshot-Direct/Reverse measurement screen will reopen.

  - Make sure your instrument is in FL aimed and centred on your Prism.

  - Do not change the point number!

  - Because the point increment is set to zero, you can just click measure for the second set of Direct/Reverse (FL/FR) readings

A Point Check screen will appear as you have measured a duplicate point. This is ok!

  - Select Store as checkshot?

  - **IMPORTANT** You MUST select the tick box Used in weighted average

    - This feature will include this second set of observations with the first as if you were completing multiple sets.

  - Select the Green Tick

You will again be returned to the Sideshot measurement screen.

  - Repeat one more set of observations as per steps 6 & 7 above.

  - This will ensure you have one Direct/Reverse Sideshot observation and two Direct/Reverse check shots included in the weighted result to the same point.

You will be presented with the Weighted Average screen once you have completed the second check shot.

  - Check that there are no unexpected Residuals in any of the observations.

  - If there is one or more erroneous observations, you will need to take additional check shots (in Sideshot mode) to ensure you have three good observations.

    - Ensure any erroneous observations are set to not be included in the weighted average.

  - If the three observations are ok, click the Green Tick to be returned to the measurement screen.

Move the Prism to the Foresight mark.

In the Sideshot measurement screen:

  - Turn the instrument in FL to the Foresight prism, ensuring the crosshairs are centred on the prism.

  - Ensure you have updated the point number appropriately.

  - Repeat steps 3 through 8 from above, to take the Foresight readings. Ensure you have three good readings to the foresight mark for your levelling observations.

## Observing the Foresight

- Once you have completed your Level observations to the Foresight station, change into Traverse measurement mode

  - To change to Traverse mode, select the button in the top left (this will be an M in Magnet branded versions, or a T in Topcon branded versions), then select Measure from the drop-down menu, and select Traverse.

  - Enter your Foresight mark number.

You will receive a Warning screen telling you the point has not been found. Click Next.

  - On the Add Point screen, enter arbitrary values for East, North and Elevation. You can take a photo of your set up using the Photo tab if you like. Then click the Green Tick button.

You will be returned to the measurement screen.

  - Ensure it is still in Traverse  mode at the top of the screen.

  - You will now be prompted to observe your Foresight Direct (FL).

  - Ensure the Total Station crosshairs are still on the prism at the Foresight.

  - Press the Measure button in the bottom right (the Total Station), or the measure and save button (Total Station with the green tick).

You will then be prompted to measure the Foresight in Reverse (FR).

  - If you are using ATR, the instrument will automatically turn to FR and aim at the prism. Check the crosshairs are centred on the prism using the telescope prior to undertaking any measurements.

  - If you are not using ATR, turn into FR and centre on the prism.

  - Press the Measure button in the bottom right (the Total Station), or the measure and save button (Total Station with the green tick).

  - The Foresight – Direct/Reverse screen will open, giving you the information from your Foresight observations. If they are acceptable, click Accept.

The program will now prompt you to move the instrument. Select Yes.

## Moving to a new Occupation

Power down the instrument and move it safely to your next occupation point, approximately halfway between your next Backsight and Foresight marks.

  - Do NOT move the prism at this point.

Power on the instrument and reconnect it to the controller.

  - In Topo, you will be prompted to enter your occupation and Backsight information again.

  - Enter the information for your new occupation as per previous steps.

  - Enter the Backsight information as per previous steps.

  - Ensure you are in Traverse measurement mode, observe the Backsight Direct/Reverse measurements.

Swap into Sideshot mode and repeat the above processes to capture your level data.

Repeat the above processes for the remainder of the survey until it is complete.

## Downloading your job file

- Shut down Topcon Field on the controller.

- Insert a USB stick into the controller.

- Navigate to the Topcon Field Jobs folder in Windows Explorer

- Copy and paste the entire Job file onto the USB

  - Note if Topcon Field is still open, there will be a Lock file that will prevent you from importing the job file into Topcon Tools later.

# Uploading TS Differential Levelling to Topcon Tools

Notes: As most software doesn’t currently have a field option for collecting differential heights with a TS, most software will also lack an easy import option. This document outlines how to get ‘clean’ your data so it can be imported as differential levels into Topcon Tools. For other software, please see the GitHub repository.

Some of the screenshots below may vary slightly depending on which version of Topcon/Magnet Tools you are using.

## Initial Import into Topcon Tools

- Import the job file to Topcon Tools as normal.

  - Note that Topcon will treat your data as a traverse, but as you didn’t set BS/FS or coordinates like normal, it may have a fit that your data is a mess. Don’t panic, this is just a through step to get the data exported into a usable format.

- Go to Report Configuration

  - Create a new report called “TS Levelling” as shown below.

  - Select “TS Observations” from the “Report item templates” list and push the double arrow button to add it “Included report items” side

- Double click on the “TS Observations” to open the menu, as shown in below.

  - Select the options as shown below.

  - Note if you did not use averaged/weighted measurements in the field this menu will look different to the figures below.

- Click on the Columns tab, as shown below

  - Select the options as shown below

- Press “Apply” and then “Ok”

- Once returned to the main report menu, you can now execute the “TS Levelling” report, ensuring you have selected the Microsoft Excel export option.

- This report can now be opened in MS Excel or similar, and you can move to the next process.

## Averaging observations in a Spreadsheet

- Depending on which field process you have selected, your observations may have been averaged already.

  - If your observations have not been averaged yet, you will need to do this within the spreadsheet.

To determine the differential height between stations using a Total Station, we must use Foresight – Backsight. This is the opposite to levelling with an auto or digi level so make sure you double check you have done it correctly!

## Example calculations

The raw data below in Table 3.4.1 is from a four (4) station level traverse using a single pole, collected at the UniSQ Springfield campus. It is provided as an example of how to reduce the collected data. No adjustment has been made.

| FORWARD RUN |  |  |  | Date | 15/09/23 | Project | UniSQ SF |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B.S. | Int. | F.S. | Δ Ht (FS-BS) | Rise | Fall | R.L | Remarks |
| -0.469 |  |  |  |  |  | 100.000 | SF3041 |
| 0.044 |  | 0.483 | 0.952 | 0.952 |  | 100.952 | SF3040 |
| 0.371 |  | 0.004 | -0.040 |  | -0.040 | 100.912 | 4000 OIP |
|  |  | 0.120 | -0.251 |  | -0.251 | 100.661 | F108 |
| Σ=-0.054 |  | Σ=0.607 |  | 0.952 | -0.291 |  |  |
| ΣFS-ΣBS= | 0.661 |  | ΣRISE-ΣFALL= | 0.661 | ΔRL= | -0.661 |  |
| REVERSE RUN |  |  |  | Date | 15/09/23 | Project | UniSQ SF |
| B.S. | Int. | F.S. | Δ Ht (FS-BS) | Rise | Fall | R.L | Remarks |
| 0.113 |  |  |  |  |  | 100.661 | F108 |
| -0.010 |  | 0.363 | 0.250 | 0.250 |  | 100.911 | 4000 OIP |
| 0.455 |  | 0.032 | 0.042 | 0.042 |  | 100.953 | SF3040 |
|  |  | -0.498 | -0.953 |  | -0.953 | 100.000 | SF3041 |
| Σ=0.558 |  | Σ=-0.103 |  | 0.292 | -0.953 |  |  |
| ΣFS-ΣBS= | -0.661 |  | ΣRISE-ΣFALL= | -0.661 | ΔRL= | 0.661 |  |

Table 3.4.1: Example data using a single pole for a level traverse of four (4) marks

## Entering your level data into Topcon Tools

- Once you have calculated the rise and fall values, completed sum checks and confirmed the data is ok, you can move to entering the data into Magnet Tools using the Manual DL Run function as shown below.

Instructions explaining how to use this function are included in Appendix A.

# Appendix A: Manual Entry of Levelling Data into Magnet/Topcon Tools

END OF DOCUMENT